# References

Aegis Engineering Repository\Monita-ftp.doc V1.6 27/01/2009

# Overview

This document describes the FTP implementation of the two-way comms feature for the DataCell RTX data loggers, and how the servers need to be set up to support this feature.

An FTP server running under either Windows or Linux will be configured to control “in” and “out” subdirectories for all communications with the loggers.  This server can be isolated and quarantined to maintain network security.

Copies of the two FTP directories will also be maintained on a server within the firewall.  A script will run periodically on this server to copy files back and forth.  This server is referred to below as the “command server”, and can be run as either Windows or Linux.

The FTP server does not initiate any actions: loggers call in to it from outside, while the command server script copies files back and forth behind the firewall.

Another option is to use a single combined server with the Command software directly accessing the FTP directories.  This is a simpler implementation, but comes at the cost of reduced security.

# Basic Data Flow

The basic flow of data for a communications session is as follows.  For this example, the logger has the serial number 12345S678.

- The Command software running under Windows is used to generate a command file for a particular logger (12345S678.cmd) based on a stored configuration file (12345S678.ini).

- The command file is written to the out directory of the command server (out/12345S678.cmd).

- A few minutes later, the script daemon running on the command server uses rsync to copy the command file to the out directory on the FTP server.

- After logger 12345S678 makes its next routine transmission, it logs in to the FTP server and checks to see if its command file is present.  This lookup would typically happen on a daily or weekly basis.

- The logger performs an FTP transfer and downloads the command file.

- The logger performs any actions required by the command file.

- The logger generates and uploads a response file back to the in directory of the FTP server (in/12345S678.ok).

- A few minutes later, the script daemon running on the command server uses rsync to copy the response file from the FTP server to the in directory on the command server, and deletes the file on the FTP server.  The daemon will then delete the original command file to prevent the logger from reading and processing it again.

- Sometime later, the Command software is used to check on the command server for the presence of the response file.  It processes this file then deletes it, completing the whole cycle.

There are two possible response files, with the suffixes .ok and .fail.  The handling of .fail files is identical to .ok fails.

The command server script also checks the integrity of files on the servers, and deletes any spurious files found.

# FTP Server Setup

Any basic FTP server can be used, running on either Windows or Unix/Linux.

The FTP server needs to have two subdirectories under its main directory: “out” and “in”.  The out directory should be set up as read-only (for FTP), and the in directory as write-only.  No files will be read from or written to the main FTP directory, so this can be set to read-only or possibly execute-only (my Unix is a bit rusty here)

The loggers will only access a restricted set of filenames (9 digits followed by .cmd, .ok or .fail) using relative pathnames off the FTP directory.  All text in the filenames should be in lower-case.

“out/12345S678.cmd”

“in/12345S678.ok”

“in/12345S678.fail”

The loggers will only attempt to read “.cmd” files, and write (create) “.ok” and “.fail” files.  They will never attempt to perform directory listings, change directories, read back “.ok” files, rename or delete files, etc.

Maximum anticipated file sizes would be 128K, even allowing for possible future enhancements.  FTP transfer types used will always be “binary” & “file”.  The loggers can be set up to use either active or passive mode transfers.

Each logger will store 12-character Userid, Password and Account strings in flash memory, and execute the corresponding FTP commands during login if they are present.  Which of these are actually used is up to the customer.  These parameters could be made unique to each logger, but this is likely to significantly complicate profile management.

The loggers will call into an FTP server at an IP address or DNS name stored as a parameter in flash memory, limited to a maximum of 30 characters.  The port number for the initial connection is stored in the logger, and defaults to the usual value of 21.

If any of the FTP access parameters (Userid, Password, Account, DNS, port number, passive mode) are changed, the logger will back up the old values.  If a future attempt to access the FTP server using the new parameters fails, a retry attempt will be made using the old parameters.  If and when an access attempt using the new parameters finally succeeds, the backup copies are deleted.

We suggest creating a group containing two FTP accounts. One account is used for the loggers while the other is used for admin/rsync. The admin account should have full access to both in and out directories.

# Rsync Setup

We recommend using rsync to handle file copying between the command and FTP servers.  Rsync will be run on the command server.  Other equivalent programs could be used if preferred.

For outgoing traffic, rsync should be set up to mirror the command server “out” directory to the FTP server “out” directory.  For example:

c:\cygwin\bin\rsync.exe -az --password-file=c:\cygwin\secret --delete /cygdrive/c/ftp/out

remotebackup@x.x.x.x::logger > logfiles\logger.log

For incoming traffic, rsync should be set up to copy all incoming files from the FTP “in” directory to the command server “in” directory, then delete these files from the FTP server.  For example:

c:\cygwin\bin\rsync.exe -az --password-file=c:\cygwin\secret --remove-source-files remotebackup@x.x.x.x::logger/in

/cygdrive/c/ftp  >> logfiles\logger.log

Install rsync client on the Command server, either using the package manager for your Linux distribution or using the Cygwin utility for Windows.  The –remove-source-files is a feature for the later versions of rsync, so please ensure your software is up to date.

Create an entry in your rsyncd.conf file to incorporate the ftp directory as above. For example:

hosts allow = x.x.x.x/x

use chroot = yes

max connections = 4

syslog facility = local5

pid file = /var/run/rsyncd.pid

[logger]

uid = loggeradmin

gid = loggeradmin

hosts allow = x.x.x.x/x

use chroot = yes

max connections = 4

read only = false

path = /home/ftp

comment = Logging FTP Server

auth users = remotebackup

secrets file = /etc/rsyncd.secrets

# Command Server Setup

The command server needs to have two transfer directories equivalent to those on the FTP server: “in” and “out”.  These need to be made available as shares to the Windows PC running the Command software, with full read/write access to both directories.

# Firewall Considerations

Since the rsync client is hosted on the Command server, and the rsync server is on the FTP server, only one connection is needed from the Command server outbound to the FTP server. No inbound connections are required. The rsync traffic can also be channelled over SSH for extra security.

The loggers can be set up to use either active or passive mode FTP.

# The Script

The script needs to be run periodically on the command server.  Running once every 5 or 10 minutes seems reasonable, since each logger will call in at most once per day.

We have provided Windows batch file and Unix bash file versions of the script.

The script also requires the use of a small program written in C to perform integrity checks on the various .cmd, .ok and .fail files, and compare internal timestamps.  This program was compiled and tested using Borland C 5.5 under Windows, and cc and gcc on a Linux machine.  Full source code is supplied.

The script performs the following tasks:

- check all files in the command server “out” directory, deleting anything other than valid .cmd files

- use rsync to mirror the command server ”out” directory to the FTP server “out” directory

- use rsync to copy all new files from the FTP server “in” directory to the command server “in” directory, then delete the original files from the FTP server

- check all files in the command server “in” directory, deleting anything other than valid .ok and .fail files

- check the command server “in” directory, and if both .ok and .fail files exist for the same serial number, delete the oldest (based on internal timestamps in the files).  If the timestamps are the same, delete the .fail file

- check the command server “in” directory against the command server “out” directory, so if both .cmd and .ok/.fail files exist for the same serial number, compare the internal timestamps and delete the oldest file.  If the timestamps are the same, delete the .cmd file.

# History

| V1.0 | Initial Release |
| --- | --- |